DNS vs Port Mapping vs NAT

Just learn these three networking concepts before your next job interview. Number one, DNS. What happens when you type google.com? So here, your browser doesn't understand names. It needs an IP address. Think of it like phone number associated to servers, and we humans cannot memorize IP addresses. We need names. When you're at google.com, it first looks at the local cache. If not found, it queries the DNS resolver. This resolver then finds the authoritative server, which could be like a .com or a root server that stores all the DNS records like a phone book. Ultimately, this server returns an IP. So, DNS is basically translating names to IP addresses. Number two, ports and port mappings. How does your browser know which service to connect to on that server? Your browser connects to an IP address, but it doesn't connect randomly. It connects to a specific port. For example, port 80 for HTTP web traffic, port 443 for HTTPS encrypted traffic, and SSH for remote login. All these are ports. So when they ask about ports, what they are testing is whether you understand that multiple services can run on the same machine using different ports. And then the port mappings. As the name suggests, it maps an external port to an internal port. It basically sets a rule that if a request is coming to this IP on this port, route it to this internal port where your app is listening. So, IP identifies the machine, port identifies the service. Number third, NAT, aka Network Address Translation. How does your private IP or machine communicates with the internet? The answer to that is NAT. Your machine that's running the application likely has a private IP address that cannot directly talk to the internet. Now think multiple applications trying to connect to the internet all at once. So there comes this process called network address translation, where a router performs the NAT. It translates your private IP address into a public IP address and the internet only sees a public IP address. Think of it like in your building, you have multiple extensions, but you have just one number. Whenever a call comes to the number, it gets routed to the right extension, keeping your private machine secure. So in one request, DNS finds the server, port finds the service and maps to it, and NAT enables private to public communication. That's the full networking part.

You’re very close, but small corrections 👍 Let’s clear it up.

✅ Correct Flow

Application (HTTP)

↓

Transport (TCP)  ← handles ports

↓

Network (IP)     ← handles routing to machine

↓

Data Link

✅ What Each Layer Does (Simple)

🟢 Network Layer (IP Layer)

👉 Finds the destination machine (IP address)
👉 Routes packets across networks
👉 Does NOT handle ports

Example:

IP: 192.168.1.10

Network layer only cares about IP.

🟢 Transport Layer (TCP/UDP)

👉 Handles ports
👉 Ensures delivery
👉 Splits data into packets
👉 Reassembles packets

Example:

Port 443 → HTTPS service

Port 80  → HTTP service

Ports are transport-layer concept.

✅ So Your Statement (Correction)

Network layer finds IP → Transport layer (TCP) handles ports

Not:

Network layer sends data to correct port

Ports are handled by TCP (transport layer).

Network layer only routes to machine (IP).

✅ Easy Interview Answer

- Network layer → IP routing

- Transport layer → ports and reliable delivery

- Application layer → HTTP request/response

🧠 Analogy

- Network layer = city address

- Transport layer = apartment number

- Application = person inside apartment

Both needed:

- Address (IP)

- Apartment (port)

If you want:

✔ Layer-by-layer deep dive
✔ Interview Q&A
✔ Spring Boot networking perspective
✔ Real packet example 🚀